All-optical switching in a nonlinear resonator

G S McDONALD and W J FIRTH

Department of Physics and Applied Physics

Strathclyde University, Glasgow G4 0NG, Scotland

Abstract.  In this paper we consider some features of all-optical switching in a unidirectional ring cavity which is partially filled with a fast and saturably nonlinear medium.  A comparative study of cross-talk for both signs of nonlinearity, in a two pixel (whole-beam switching) configuration, is made.  A particular interaction modulation is discovered for two beams in the self-defocusing case.  For self-defocusing media, results are extended by consideration of large (part-beam switched) solitary arrays.  Prescribed binary patterns may be stably encoded in a single cavity transit which is in sharp contrast to the hundreds of transits required for spontaneous stabilisation.  These patterns are seen to be stable over thousands of transits.